Prusa Conversion Guide, v0.1

Parts to Print

Settings: 50% infill, 0.16 mm Creality, 0.15 mm Mini+, 0.15 mm Ultimaker

Replistruder4_Core.stl – 16:30 / 12:27 / 9:15

Mount_v2 v3.stl – 1:40 / 1:47 / 2:00

2x Replistruder4_CoreSupport_Modified v2.stl – 1:35 / 1:10 / 1:21

Top_Plate_shifterbolts_v2 v2.stl – 0:44 / 0:36 / 0:34

Bottom_Plate_shiftedbolts_v2 v0.stl – 1:00 / 0:56 / 0:44

Version2_NutSide v5.stl – 2:37 / 1:33 / 1:33

Version2_ScrewSide v5 – 2:30 / 1:33 / 1:32

Mini+_allparts_nocore.gcode – 8:44

UM2_Replistruder4_Core – 9:13

Firmware Modification:

Download: Prusa-Firmware-Buddy-master

Modify: ./include/marlin/Configuration_MINI_adv.h

Change line 988 to:

#define Z_MIN_POS -80

Change lines 493 and 494:

#define PREVENT_COLD_EXTRUSION

#define EXTRUDE_MINTEMP 0

Run:

python3 ./utils/build.py --generate-bbf

Note that you will have permission errors if you copy files compiled on one computer to run them on another

Now copy:

/build/products/mini_release_boot.bbf

To a USB drive

This should be read automatically (you don't need to hit any buttons) - there will be a warning about how the key for the firmware is invalid (hit ignore).

Converting Your Prusa Mini+ to a Liquid/Gel FrankenPrinter

Part 1 – Gutting your Beloved Printer

You’ll need:

Pliers

2.5 mm Allen / Hex Wrench

Small flathead screwdriver

Steps

- Unload any filament already loaded in the printer, then switch off and wait for everything to cool down.

- Unscrew the large brass bolt from the extruder mount and remove the plastic tube that usually carries the filament.

- Open up the electronics enclosure and remove plugs 1-5, leaving the black/blue/red/green extruder stepper motor plugged in.

- While you’re here – it’s time for the scary part of the operation. You need to physically break a connection on the motherboard to be able to install custom firmware. Take a small, flathead screwdriver and break the part of the logic board shown in the photo below. Apply a reasonable amount of force to the thinnest bit of the board – you should end up with something that looks like this:

Before doing this, you may want to consult an emotional support website – either a more practical one:

https://help.prusa3d.com/article/flashing-custom-firmware-mini_14

Or simply some pictures of puppies.

- Cut the 3 cable ties attached to the extruder stepper mount and remove the wires from the protective sheath, and gently remove the protective sheath.

- Place the cable from the stepper motor back in the protective sheath and replace the motherboard cover and the screws that hold it in place. You should have something that looks like this:

- Now it’s time to remove the hot-end. Remove the screws shown by the white below (it is almost certainly possible to achieve this while removing fewer screws).

- Finally, (1.) pull the fan out – this might take a bit of force and (2.) remove the final screw attaching the hot end.

- You should be left with something that looks like the photo below – well done! You’re ready to assemble and mount your new printhead!

Part 2 – Assembling Your Forklistruder_Mod Printhead